Yorick Documentation

for functions, variables, and structures

defined in file std.i

Printed: Fri Mar  9 06:23:45 2007

Contents:

_                   catch               interp              rmdir

__alpha             cd                  is_array            save

__cray              ceil                is_func             sech

__dec               close               is_range            set_blocksize

__i86               close102            is_stream           set_filesize

__ibmpc             close102_default    is_struct           set_idler

__mac               collect             is_void             set_path

__macl              conj                jc                  set_primitives

__sgi64             copyright           jr                  set_vars

__sun               cos                 jt                  sgi64_primitives

__sun3              cosh                legal               show

__vax               cray_primitives     library             sign

__vaxg              create              log                 sin

__xdr               createb             log10               sinh

_car                data_align          lsdir               sizeof

_cat                dbauto              mac_primitives      sort

_cdr                dbcont              macl_primitives     span

_cpy                dbdis               max                 spanl

_init_clog          dbexit              median              sqrt

_init_pdb           dbinfo              merge               sread

_jc                 dbret               merge2              strlen

_jr                 dbskip              min                 strmatch

_jt                 dbup                mkdir               strpart

_len                dec_primitives      nallof              strtok

_lst                digitize            nameof              struct_align

_map                dimsof              noneof              structof

_not_cdf            disassemble         numberof            sum

_not_pdb            dump_clog           open                sun3_primitives

_nxt                edit_times          open102             sun_primitives

_prt                eq_nocopy           openb               swrite

_read               error               openb_hooks         symbol_def

_rev                exit                orgsof              symbol_set

_set_pdb            exp                 pc_primitives       system

_write              fflush              pi                  tan

abs                 floor               poly                tanh

acos                get_addrs           popen               timer

add_member          get_argv            pr1                 timer_print

add_next_file       get_cwd             print               timestamp

add_record          get_env             print_format        transpose

add_variable        get_home            process_argv        typeof

allof               get_member          ptcen               uncen

alpha_primitives    get_ncycs           quit                updateb

am_subroutine       get_primitives      random              use_origins

anyof               get_times           random_seed         vax_primitives

area                get_vars            randomize           vaxg_primitives

array               grow                rdline              volume

asin                has_records         re_part             warranty

asinh               help                read                where

at_pdb_close        histogram           read_clog           where2

at_pdb_open         i86_primitives      read_n              write

atan                im_part             recover_file        xdr_primitives

avg                 include             remove              yorick_stats

backup              indgen              rename              zncen

batch               info                require

bookmark            install_struct      reshape

call                integ               restore

FROM _ TO __xdr

_

/* SEE grow     */

__alpha

/* SEE __xdr     */

__cray

/* SEE __xdr     */

__dec

/* SEE __xdr     */

__i86

/* SEE __xdr     */

__ibmpc

/* SEE __xdr     */

__mac

/* SEE __xdr     */

__macl

/* SEE __xdr     */

__sgi64

/* SEE __xdr     */

__sun

/* SEE __xdr     */

__sun3

/* SEE __xdr     */

__vax

/* SEE __xdr     */

__vaxg

/* SEE __xdr     */

__xdr

/* DOCUMENT primitive data types for various machines:

little-endians

__i86      Intel x86 Linux

__ibmpc    IBM PC (2 byte int)

__alpha    Compaq alpha

__dec      DEC workstation (MIPS), Intel x86 Windows

__vax      DEC VAX (H-double)

__vaxg     DEC VAX (G-double)

big-endians

__xdr      External Data Representation

__sun      Sun, HP, SGI, IBM-RS6000, MIPS 32 bit

__sun3     Sun-2 or Sun-3 (old)

__sgi64    SGI, Sun, HP, IBM-RS6000 64 bit

FROM __xdr TO _len

__mac      MacIntosh 68000 (power Mac, Gx are __sun)

__macl     MacIntosh 68000 (12 byte double)

__cray     Cray XMP, YMP

SEE ALSO: set_primitives

*/

_car

/* SEE _lst     */

_cat

/* SEE _lst     */

_cdr

/* SEE _lst     */

_cpy

/* SEE _lst     */

_init_clog

/* DOCUMENT _init_clog, file

initializes a Clog binary file.  Used after creating a new file --

must be called AFTER the primitive data formats have been set.

*/

_init_pdb

/* DOCUMENT _init_pdb, file, at_pdb_close

_set_pdb, file, at_pdb_close

initializes a PDB binary file.  Used after creating a new file --

must be called AFTER the primitive data formats have been set.

The _set_pdb call only sets the CloseHook, on the assumption that

the file header has already been written (as in recover_file).

SEE ALSO: createb, recover_file, at_pdb_close

*/

_jc

/* SEE _jr     */

_jr

/* DOCUMENT _jt, file, time

_jc, file, ncyc

_jr, file

are raw versions of jt and jc provided to simplify redefining

the default jt and jc functions to add additional features.

For example, you could redefine jt to jump to a time, then

plot something.  The new jt can pass its arguments along to

_jt, then call the appropriate plotting functions.

There is a raw version of jr as well.

*/

_jt

/* SEE _jr     */

_len

/* SEE _lst     */

FROM _lst TO _lst

_lst

/* DOCUMENT list= _lst(item1, item2, item3, ...)

list= _cat(item_or_list1, item_or_list2, item_or_list3, ...)

list= _cpy(list)

list= _cpy(list, i)

length= _len(list)

item= _car(list)

item_i= _car(list, i)

_car, list, i, new_item_i

list= _cdr(list)

list= _cdr(list, i)

_cdr, list, i, new_list_i

implement rudimentary Lisp-like list handling in Yorick.

However, in Yorick, a list must have a simple tree structure

- no loops or rings are allowed (loops break Yorick's memory

manager - beware).  You need to be careful not to do this as

the error will not be detected.

Lists are required in Yorick whenever you need to hold an

indeterminate amount of non-array data, such as file handles,

bookmarks, functions, index ranges, etc.  Note that Yorick

pointers cannot point to these objects.  For array data, you have

a choice between a list and a struct or an array of pointers.

Note that a list cannot be written into a file with the save

function, since it may contain unsaveable items.

The _lst (list), _cat (catenate), and _cpy (copy) functions

are the principal means for creating and maintaining lists.

_lst makes a list out of its arguments, so that each argument

becomes one item of the new list.  Unlike Yorick array data

types, a statement like x=list does not make a copy of the

list, it merely makes an additional reference to the list.

You must explicitly use the _cpy function to copy a list.  Note

that _cpy only copies the outermost list itself, not the items

in the list (even if those items are lists).  With the second

argument i, _cpy copies only the first i items in the list.

The _cat function concatentates several lists together,

"promoting" any arguments which are not lists.  This operation

changes the values of list arguments to _cat, except for the

final argument, since after _cat(list, item), the variable list

will point to the new longer list returned by _cat.

Nil, or [], functions as an empty list.  This leads to ambiguity

in the argument list for _cat, since _cat "promotes" non-list

arguments to lists; _cat treats [] as an empty list, not as a

non-list item.  Also, _lst() or _lst([]) returns a single item list,

not [] itself.

The _len function returns the number of items in a list, or 0

for [].

The _car and _cdr functions (the names are taken from Lisp,

where they originally stood for something like "address register"

and "data register" of some long forgotten machine) provide

FROM _lst TO _nxt

access to the items stored in a list.  _car(list,i) returns the

i-th item of the list, and i defaults to 1, so _car(list) is the

first item.  Also, _car,list,i,new_item_i sets the i-th item

of the list.  Finally, _cdr(list,i) returns a list of all the

items beyond the i-th, where i again defaults to 1.  The form

_cdr,list,i,new_list_i can be used to reset all list items

beyond the i-th to new values.  In the _cdr function, i=0 is

allowed.  When used to set values, both _car and _cdr can also

be called as functions, in which case they return the item or

list which has been replaced.  The _cdr(list) function returns

nil if and only if LIST contains only a single item; this is

the usual means of halting a loop over items in a list.

SEE ALSO: array, grow, _prt, _map, _rev, _nxt

*/

_map

/* DOCUMENT _map(f, list)

return a list of the results of applying function F to each

element of the input LIST in turn, as if by

_lst(f(_car(list,1)),f(_car(list,2)),...)

SEE ALSO: _lst

*/

_not_cdf

/* DOCUMENT _not_cdf(file)

is like _not_pdb, but for netCDF files.

*/

_not_pdb

/* DOCUMENT _not_pdb(file, familyOK)

returns 1 if FILE is not a PDB file, otherwise returns 0 after

setting the structure and data tables, and cataloguing any

history records.  Used to open an existing file.  Also detects

a file with an appended Clog description.

Before calling _not_pdb, set the variable yPDBopen to the value

of at_pdb_open you want to be in force.  (For historical reasons

-- in order to allow for the open102 keyword to openb -- _not_pdb

looks at the value of the variable yPDBopen, rather than at_pdb_open

directly.)

*/

_nxt

/* DOCUMENT item= _nxt(list)

return first item in LIST, and set LIST to list of remaining

items.  If you are iterating through a list, this is the way

to do it, since a loop on _car(list,i) with i varying from 1

to _len(list) scales quadratically with the length of the list,

while a loop on _nxt(list) scales linearly.

SEE ALSO: _car, _lst

*/

FROM _prt TO acos

_prt

/* DOCUMENT _prt, list

print every item in a list, recursing if some item is itself a list.

SEE ALSO: _lst

*/

_read

/* DOCUMENT _write, file, address, expression

_read, file, address, variable

or nbytes= _read(file, address, variable);

are low level read and write functions which do not "see" the

symbol table for the binary FILE.  The ADDRESS is the byte address

at which to begin the write or read operation.  The type and number

of objects of the EXPRESSION or VARIABLE determines how much data

to read, and what format conversion operations to apply.  In the

case of type char, no conversion operations are ever applied, and

_read will return the actual number of bytes read, which may be

fewer than the number implied by VARIABLE in this one case.

(In all other cases, _read returns numberof(VARIABLE).)

If the FILE has records, the ADDRESS is understood to be in the

file family member in which the current record resides.

SEE ALSO: openb, createb, updateb, save, restore, sizeof

*/

_rev

/* DOCUMENT _rev(list)

returns the input list in reverse order

SEE ALSO: _lst

*/

_set_pdb

/* SEE _init_pdb     */

_write

/* SEE _read     */

abs

/* DOCUMENT abs(x)

or abs(x, y, z, ...)

returns the absolute value of its argument.

In the multi-argument form, returns sqrt(x^2+y^2+z^2+...).

SEE ALSO: sign, sqrt

*/

acos

/* DOCUMENT acos(x)

returns the inverse cosine of its argument, range [0, pi].

SEE ALSO: sin, cos, tan, asin, acos, atan

*/

FROM add_member TO add_record

add_member

/* DOCUMENT add_member, file, struct_name, offset, name, type, dimlist

adds a member to a data type in the file FILE.  The data type name

(struct name) is STRUCT_NAME, which will be created if it does

not already exist.  The new member will be at OFFSET (in bytes)

from the beginning of an instance of this structure, and will

have the specified NAME, TYPE, and DIMLIST.  Use OFFSET -1 to

have add_member compute the next available offset in the structure.

The TYPE can be either a structure definition, or a string naming

a previously defined data type in FILE.  The optional DIMLIST is

as for the "array" function.

The STRUCT_NAME built from a series of add_member calls cannot be

used until it is installed with install_struct.

This function should be used very sparingly, mostly in code which

is building the structure of a foreign-format binary file.

SEE ALSO: add_variable, install_struct, struct_align

*/

add_next_file

/* DOCUMENT failure= add_next_file(file, filename, create_flag)

adds the next file to the FILE, which must contain history records.

If FILENAME is non-nil, the new file will be called that, otherwise

the next sequential filename is used.  If CREATE_FLAG is present

and non-zero, the new file will be created if it does not already

exist.  If omitted or nil, CREATE_FLAG defaults to 1 if the file has

write permission and 0 if it does not.

Returns 0 on success.

SEE ALSO: openb, updateb, createb, add_record

*/

add_record

/* DOCUMENT add_record, file, time, ncyc

or add_record, file, time, ncyc, address

or add_record, file

adds a new record to FILE corresponding to the specified TIME and

NCYC (respectively a double and a long).  Either or both TIME

and NCYC may be nil or omitted, but the existence of TIME and

NCYC must be the same for every record added to one FILE.

If present, ADDRESS specifies the disk address of the new record,

which is assumed to be in the current file.  Without ADDRESS, or

if ADDRESS<0, the next available address is used; this may create

a new file in the family (see the set_filesize function).

The add_record function leaves the new record current

for subsequent save commands to actually write the data.

The TIME, NCYC, and ADDRESS arguments may be equal length vectors

to add several records at once; in this case, the first of the

newly added records is the current one.  If all three of TIME,

NCYC, and ADDRESS are nil or omitted, no new records are added,

but the file becomes a record file if it was not already, and in

any case, no record will be the current record after such an

add_record call.

After the first add_record call (even if no records were added),

subsequent add_variable commands will create record variables.

FROM add_record TO anyof

After the first record has been added, subsequent save commands

will create any new variables as record variables.

After a second record has been added using add_record, neither

save commands nor add_variable commands may be used to introduce

any new record variables.

SEE ALSO: save, createb, updateb, openb, set_filesize, set_blocksize

add_variable

*/

add_variable

/* DOCUMENT add_variable, file, address, name, type, dimlist

adds a variable NAME to FILE at the specified ADDRESS, with the

specified TYPE and dimensions given by DIMLIST.  The DIMLIST may

be zero or more arguments, as for the "array" function.  If the

ADDRESS is <0, the next available address is used. Note that,

unlike the save command, add_variable does not actually write any

data -- it merely changes Yorick's description of the contents of

FILE.

After the first add_record call, add_variable adds a variable to

the record instead of a non-record variable.  See add_record.

SEE ALSO: save, openb, createb, updateb, add_record,

add_member, install_struct, data_align

*/

allof

/* DOCUMENT allof(x)

anyof(x)

nallof(x)

noneof(x)

Respectively:

returns 1 if every element of the array x is non-zero, else 0.

returns 1 if at least one element of the array x is non-zero, else 0.

returns 1 if at least one element of the array x is zero, else 0.

returns 1 if every element of the array x is zero, else 0.

SEE ALSO: allof, anyof, noneof, nallof, where, where2

*/

alpha_primitives

/* DOCUMENT alpha_primitives, file

sets FILE primitive data types to be native to DEC alpha workstations.

*/

am_subroutine

/* DOCUMENT am_subroutine()

returns 1 if the current Yorick function was invoked as a subroutine,

else 0.  If am_subroutine() returns true, the result of the current

function will not be used, and need not be computed (the function

has been called for its side effects only).

*/

anyof

/* SEE allof     */

FROM area TO at_pdb_open

area

/* DOCUMENT area(y, x)

returns the zonal areas of the 2-D mesh (X, Y).  If Y and X are

imax-by-jmax, the result is (imax-1)-by-(jmax-1).  The area is

positive when, say, X increases with i and Y increases with j.

For example, area([[0,0],[1,1]],[[0,1],[0,1]]) is +1.

SEE ALSO: volume

*/

array

/* DOCUMENT array(value, dimension_list)

or array(type, dimension_list)

returns an object of the same type as VALUE, consisting of copies

of VALUE, with the given DIMENSION_LIST appended to the dimensions

of VALUE.  Hence, array(1.5, 3, 1) is the same as [[1.5, 1.5, 1.5]].

In the second form, the VALUE is taken as scalar zero of the TYPE.

Hence, array(short, 2, 3) is the same as [[0s,0s],[0s,0s],[0s,0s]].

A DIMENSION_LIST is a list of arguments, each of which may be

any of the following:

(1) A positive scalar integer expression,

(2) An index range with no step field (e.g.-  1:10), or

(3) A vector of integers [number of dims, length1, length2, ...]

(that is, the format returned by the dimsof function).

SEE ALSO: reshape, is_array, dimsof, numberof, grow, span, use_origins,

_lst

*/

asin

/* DOCUMENT asin(x)

returns the inverse sine of its argument, range [-pi/2, pi/2].

SEE ALSO: sin, cos, tan, asin, acos, atan

*/

asinh

/* DOCUMENT asinh(x)

acosh(x)

atanh(x)

returns the inverse hyperbolic sine, cosine, or tangent of

its argument.  The range of acosh is >=0.0.

SEE ALSO: sinh, cosh, tanh, sech, csch

*/

at_pdb_close

/* SEE at_pdb_open     */

at_pdb_open

/* DOCUMENT at_pdb_open

at_pdb_close

bits for optional behavior when a PDB file is opened or closed:

at_pdb_open:

000  Major-Order:  value specified in file is correct

001  Major-Order:102 always

002  Major-Order:  opposite from what file says

003  Major-Order:101 always

FROM at_pdb_open TO batch

004  Strip Basis @... suffices from variable names (when possible)

Danger!  If you do this and open a file for update, the variable

names will be stripped when you close the file!

010  Use Basis @history convention on input

The 001 and 002 bits may be overridden by the open102 keyword.

The default value of at_pdb_open is 010.

at_pdb_close (the value at the time the file is opened or created

is remembered):

001  Write Major-Order 102 PDB file

002  Write PDB style history data

The following are no-ops unless bit 002 is set:

004  Use Basis @history convention on output

010  Do NOT pack all history record variables into

a single structure instance.

The 001 bit may be overridden by the close102 keyword or if

close102_default is non-zero.

The default value of at_pdb_close is 007.

SEE ALSO: close102_default

*/

atan

/* DOCUMENT atan(x)

or atan(y, x)

returns the inverse tangent of its argument, range [-pi/2, pi/2].

In the two argument form, returns the angle from (1, 0) to (x, y),

in the range (-pi, pi], with atan(1, 0)==pi/2.  (If x>=0, this is

the same as atan(y/x).)

SEE ALSO: sin, cos, tan, asin, acos, atan

*/

avg

/* DOCUMENT avg(x)

returns the scalar average of all elements of its array argument.

SEE ALSO: sum, min, max

*/

backup

/* SEE bookmark     */

batch

/* DOCUMENT batch, 1

batch, 0

batch()

turns on, turns off, or tests for batch mode, respectively.

If yorick is started with the command line:

yorick -batch batch_include.i ...

then batch mode is turned on, the usual custom.i startup file is

skipped, and the file batch_include.i is parsed and executed.  The

-batch and batch_include.i command line arguments are removed from

the list returned by get_argv().  These must be the first two

FROM batch TO catch

arguments on the command line.

In batch mode, any error will terminate Yorick (as by the quit

function) rather than entering debug mode.  Also, any attempt to

read from the keyboard is an error.

SEE ALSO: process_argv, get_argv, set_idler

*/

bookmark

/* DOCUMENT backup, f

or bmark= bookmark(f)

...

backup, f, bmark

back up the text stream F, so that the next call to the read

function returns the same line as the previous call to read

(note that you can only back up one line).  If the optional

second argument BMARK is supplied, restores the state of the

file F to its state at the time the bookmark function was

called.

After a matching failure in read, use the single argument form

of backup to reread the line containing the matching failure.

SEE ALSO: read, rdline, open, close

*/

call

/* DOCUMENT call, subroutine(arg1, arg2, arg3, arg4, arg5

arg6, arg7, arg8);

allows a SUBROUTINE to be called with a very long argument list

as an alternative to:

subroutine, arg1, arg2, arg3, arg4, arg5,

arg6, arg7, arg8;

Note that the statement

subroutine(arg1, arg2, arg3, arg4, arg5,

arg6, arg7, arg8);

will print the return value of subroutine, even if it is nil.

If invoked as a function, call simply returns its argument.

*/

catch

/* DOCUMENT catch(category)

Catch errors of the specified category.  Category may be -1 to

catch all errors, or a bitwise or of the following bits:

0x01 math errors (SIGFPE, math library)

0x02 I/O errors

0x04 keyboard interrupts (e.g.- control C interrupt)

0x08 other compiled errors (YError)

0x10 interpreted errors (error)

Use catch by placing it in a function before the section of code

in which you are trying to catch errors.  When catch is called,

it always returns 0, but it records the virtual machine program

counter where it was called, and longjumps there if an error is

detected.  The most recent matching call to catch will catch the

FROM catch TO ceil

error.  Returning from the function in which catch was called

pops that call off the list of catches the interpreter checks.

To use catch, place the call near the top of a function:

if (catch(category)) {

...<code to execute if error is caught>...

}

...<code "protected" by the catch>...

If an error with the specified category occurs in the "protected"

code, the program jumps back to the point of the catch and acts

as if the catch function had returned 1 (remember that when catch

is actually called it always returns 0).

In order to lessen the chances of infinite loops, the catch is

popped off the active list if it is actually used, so that a

second error will *not* be caught.  Often, this is only desirable

for the error handling code itself -- if you want to re-execute

the "protected" code, do this, and take care of the possibility

of infinite loops in your interpreted code:

while (catch(category)) {

...<code to execute if error is caught>...

}

...<code "protected" by the catch>...

After an error has been caught, the associated error message

(what would have been printed had it not been caught) is left

in the variable catch_message.

SEE ALSO: error

*/

cd

/* DOCUMENT cd, directory_name

or cd(directory_name)

change current working directory to DIRECTORY_NAME, returning

the expanded path name (i.e.- with leading environment variables,

., .., or ~ replaced by the actual pathname).  If called as a

function, returns nil to indicate failure, otherwise failure

causes a Yorick error.

SEE ALSO: lsdir, mkdir, rmdir, get_cwd, get_home, get_env, get_argv

*/

ceil

/* DOCUMENT ceil(x)

returns the smallest integer not less than x (no-op on integers).

SEE ALSO: floor

*/

FROM close TO close102

close

/* DOCUMENT close, f

closes the I/O stream F (returned earlier by the open function).

If F is a simple variable reference (as opposed to an expression),

the close function will set F to nil.  If F is the only reference

to the I/O stream, then "close, f" is equivalent to "f= []".

Otherwise, "close, f" will close the file (so that subsequent

I/O operations will fail) and print a warning message about the

outstanding ("stale") references.

SEE ALSO: open, read, write, rdline, bookmark, backup, save, restore,

rename, remove

*/

close102

/* DOCUMENT close102  is a keyword for createb or updateb,

open102   is a keyword for openb or updateb

close102_default   is a global variable (initially 0)

***Do not use close102_default -- use at_pdb_close

-- this is for backward compatibility only***

close102=1  means to close the PDB file "Major-Order:102"

close102=0  means close it "Major-Order:101"

if not specified, uses 1 if close102_default non-zero,

otherwise the value specified in at_pdb_close

open102=1   means to ignore what the PDB file says internally,

and open it as if it were "Major-Order:102"

open102=0   (the default) means to assume the PDB file is

correctly writen

open102=2   means to assume that the file is incorrectly

written, whichever way it is marked

open102=3   means to ignore what the PDB file says internally,

and open it as if it were "Major-Order:101"

The PDB file format comes in two styles, "Major-Order:101", and

"Major-Order:102".  Yorick interprets these correctly by default,

but other codes may ignore them, or write them incorrectly.

Unlike Yorick, not all codes are able to correctly read both

styles.  If you are writing a file which needs to be read by

a "102 style" code, create it with the close102=1 keyword.

If you notice that a file you though was a history file isn't, or

that the dimensions of multi-dimensional variables are transposed

from the order you expected, the code which wrote the file probably

blew it.  Try openb("filename", open102=2).  The choices 1 and 3

are for cases in which you know the writing code was supposed to

write the file one way or the other, and you don't want to be

bothered.

The open102 and close102 keywords, if present, override the

defaults in the variables at_pdb_open and at_pdb_close.

SEE ALSO: at_pdb_open, at_pdb_close

*/

FROM close102_default TO cray_primitives

close102_default

/* SEE close102     */

collect

/* DOCUMENT result= collect(f, name_string)

scans through all records of the history file F accumulating the

variable NAME_STRING into a single array with one additional

index varying from 1 to the number of records.

NAME_STRING can be either a simple variable name, or a name

followed by up to four simple indices which are either nil, an

integer, or an index range with constant limits.  (Note that

0 or negative indices count from the end of a dimension.)

Examples:

collect(f, "xle")        -- collects the variable f.xle

collect(f, "tr(2,2:)")   -- collects f.tr(2,2:)

collect(f, "akap(2,-1:0,)") -- collects f.akap(2,-1:0,)

(i.e.- akap in the last two values of its

second index)

SEE ALSO: get_times

*/

conj

/* DOCUMENT conj(z)

returns the complex conjugate of its argument.

*/

copyright

/* DOCUMENT copyright, (no) warranty

Copyright (c) 1996.  The Regents of the University of California.

All rights reserved.

Yorick is provided "as is" without any warranty, either expressed or

implied.  For a complete statement, type:

legal

at the Yorick prompt.

SEE ALSO: legal

*/

cos

/* SEE sin     */

cosh

/* SEE sinh     */

cray_primitives

/* DOCUMENT cray_primitives, file

sets FILE primitive data types to be native to Cray 1, XMP, and YMP.

*/

FROM create TO dbcont

create

/* DOCUMENT f= create(filename)

is a synonym for       f= open(filename, "w")

Creates a new text file FILENAME, destroying any existing file of

that name.  Use the write function to write into the file F.

SEE ALSO: write, close, open

*/

createb

/* DOCUMENT file= createb(filename)

or file= createb(filename, primitives)

creates FILENAME as a PDB file in "w+b" mode, destroying any

existing file by that name.  If the PRIMITIVES argument is

supplied, it must be the name of a procedure that sets the

primitive data types for the file.  The default is to create

a file with the native primitive types of the machine on which

Yorick is running.  The following PRIMITIVES functions are

predefined:

sun_primitives    -- appropriate for Sun, HP, IBM, and

most other workstations

sun3_primitives   -- appropriate for old Sun-2 or Sun-3

dec_primitives    -- appropriate for DEC (MIPS) workstations, Windows

alpha_primitives  -- appropriate for DEC alpha workstations

sgi64_primitives  -- appropriate for 64 bit SGI workstations

cray_primitives   -- appropriate for Cray 1, XMP, and YMP

mac_primitives    -- appropriate for MacIntosh

macl_primitives   -- appropriate for MacIntosh, 12-byte double

i86_primitives    -- appropriate for Linux i86 machines

pc_primitives     -- appropriate for IBM PC

vax_primitives    -- appropriate for VAXen only (H doubles)

vaxg_primitives   -- appropriate for VAXen only (G doubles)

xdr_primitives    -- appropriate for XDR files

SEE ALSO: openb, updateb, cd

save, add_record, set_filesize, set_blocksize

close102, close102_default, at_pdb_open, at_pdb_close

*/

data_align

/* DOCUMENT data_align, file, alignment

in binary file FILE, align new variables to begin at a byte address

which is a multiple of ALIGNMENT.  (This affects placement of data

declared using save and add_variable.  For add_variable, data_align

has an effect only if the address is not specified.)  If ALIGNMENT

is <=0, new variables will be aligned as they would be if they were

data structure members.  The default value is 0.

SEE ALSO: save, add_variable

*/

dbauto

/* SEE dbexit     */

dbcont

/* SEE dbexit     */

FROM dbdis TO dbexit

dbdis

/* SEE dbexit     */

dbexit

/* DOCUMENT Debug mode.

Yorick errors fall into two general categories: Syntax errors discovered

during parsing, and runtime errors discovered when a Yorick program is

actually running.  When a runtime error occurs, Yorick offers the

choice of entering "debug mode", which you can do by typing the <RETURN>

key immediately after the error occurs.  Typing a non-blank line exits

debug mode automatically by default.  In debug mode, the Yorick prompt

becomes "dbug>" instead of the usual ">".  When you see this prompt,

Yorick has halted "in the middle of" the function in which the error

occurred, and you can print, plot, modify, or save the local variables

in that function by means of ordinary Yorick commands.  Debug mode is

recursive; that is, you can debug an error which occurred during

debugging to any number of levels.

You can exit from debug mode in several ways:

dbexit            -- exit current debug level, discarding all

active functions and their local variables

dbexit, 0         -- exit all debug levels

dbexit, n         -- exit (at most) N debug levels

dbcont            -- continue execution of the current function

Continuing is useful if you have managed to repair the

problem which caused the error.  The expression in which the

error occurred will be evaluated a second time, so beware of

side effects.

dbret, value      -- continue execution by returning VALUE (which

may be nil or omitted) to the caller of the

function in which the error occurred.

This is useful if the function in which the error occurred is

hopelessly confounded, but you know the value it should return.

Yorick does not allow "single stepping" directly, although you can

execute the statements in a function by copying them, then tell

Yorick to skip those statements you have executed "by hand".  There

are two functions for skipping execution:

dbskip            -- skip the next logical line (This will be only

a portion of a source line if several statements

are stacked on the source line.)

dbskip, n         -- skip next N (positive or negative) logical lines

dbup              -- discard the current function, so that you are

debugging its caller -- there is no way to go

back "down", so be careful

There are two functions which print information (like other print

functions, if called as functions instead of subroutines, their

result is returned as a string array with one line per string):

FROM dbexit TO dimsof

dbinfo            -- returns current function and source line

dbdis             -- returns disassembled virtual machine code

for the next line (use the disassemble function

to get the entire function)

This allows you to see exactly where in a line the error occurred.

Finally,

dbauto            -- toggles whether debug mode will be entered

automatically when a runtime error occurs

dbauto, 1         -- enter debug mode automatically after an error

dbauto, 0         -- type <RETURN> after error to enter debug mode

*/

dbinfo

/* SEE dbexit     */

dbret

/* SEE dbexit     */

dbskip

/* SEE dbexit     */

dbup

/* SEE dbexit     */

dec_primitives

/* DOCUMENT dec_primitives, file

sets FILE primitive data types to be native to DEC (MIPS) workstations.

*/

digitize

/* DOCUMENT digitize(x, bins)

returns an array of longs with dimsof(X), and values i such that

BINS(i-1) <= X < BINS(i) if BINS is monotonically increasing, or

BINS(i-1) > X >= BINS(i) if BINS is monotonically decreasing.

Beyond the bounds of BINS, returns either i=1 or i=numberof(BINS)+1

as appropriate.

SEE ALSO: histogram, interp, integ, sort, where, where2

*/

dimsof

/* DOCUMENT dimsof(object)

or dimsof(object1, object2, ...)

returns a vector of integers describing the dimensions of OBJECT.

The format of the vector is [number of dims, length1, length2, ...].

The orgsof function returns the origin of each dimension (normally 1).

If more than one argument is given, dimsof returns the dimension

list of the result of binary operations between all the objects,

or nil if the objects are not conformable.

SEE ALSO: typeof, structof, numberof, sizeof, orgsof

*/

FROM disassemble TO eq_nocopy

disassemble

/* DOCUMENT disassemble(function)

or disassemble, function

Disassembles the specified function.  If called as a function, the

result is returned as a vector of strings; if called as a subroutine,

the disassembly is printed at the terminal.  If the function is nil,

the current *main* program is disassembled -- you must include the

call to disassemble in the main program, of course, NOT on its own

line as a separate main program.

*/

dump_clog

/* DOCUMENT dump_clog, file, clog_name

dumps a Contents Log of the binary file FILE into the text file

CLOG_NAME.  Any previous file named CLOG_NAME is overwritten.

SEE ALSO: openb

*/

edit_times

/* DOCUMENT edit_times, file

or edit_times, file, keep_list

or edit_times, file, keep_list, new_times, new_ncycs

edits the records for FILE.  The KEEP_LIST is a 0-origin index list

of records to be kept, or nil to keep all records.  The NEW_TIMES

array is the list of new time values for the (kept) records, and

the NEW_NCYCS array is the list of new cycle number values for the

(kept) records.  Either NEW_TIMES, or NEW_NCYCS, or both, may be

nil to leave the corresponding values unchanged.  If non-nil,

NEW_TIMES and NEW_NCYCS must have the same length as KEEP_LIST,

or, if KEEP_LIST is nil, as the original number of records in

the file.  If KEEP_LIST, NEW_TIME, and NEW_NCYCS are all omitted

or nil, then edit_times removes records as necessary to ensure

that the remaining records have monotonically increasing times,

or, if no times are present, monotonically increasing ncycs.

(The latest record at any given time/ncyc is retained, and earlier

records are removed.)

In no case does edit_times change the FILE itself; only Yorick's

in-memory model of the file is altered.

SEE ALSO: get_times, get_ncycs, jt, jc

*/

eq_nocopy

/* DOCUMENT eq_nocopy, y, x

is the same as

y= x

except that if x is an array, it is not copied, even if it is

not a temporary (i.e.- an expression).  Having multiple variables

reference the same data can be confusing, which is why the default

= operation copies the array.  The most important use of eq_nocopy

involves pointers or lists:

y= *py

z= _car(list)

always causes the data pointed to by py to be copied, while

eq_nocopy, y, *py

eq_nocopy, z, _car(list)

FROM eq_nocopy TO get_addrs

does not copy the data - often more nearly what you wanted.

Note that scalar int, long, and double variables are always copied,

so you cannot count on eq_nocopy setting up an "equivalence"

between variables.

*/

error

/* DOCUMENT exit, msg

error, msg

Exits the current interpreted *main* program, printing the MSG.

(MSG can be omitted to print a default.)

In the case of exit, the result is equivalent to an immediate

return from every function in the current calling chain.

In the case of error, the result is the same as if an error had

occurred in a compiled routine.

SEE ALSO: print, write, batch, catch

*/

exit

/* SEE error     */

exp

/* DOCUMENT exp(x)

returns the exponential function of its argument (inverse of log).

SEE ALSO: log, log10, sinh, cosh, tanh, sech, csch

*/

fflush

/* DOCUMENT fflush, file

flush the I/O buffers for the text file FILE.  (Binary files are

flushed at the proper times automatically.)  You should only need

this after a write, especially to a pipe.

SEE ALSO: write, popen

*/

floor

/* DOCUMENT floor(x)

returns the largest integer not greater than x (no-op on integers).

SEE ALSO: ceil

*/

get_addrs

/* DOCUMENT addr_lists= get_addrs(file)

returns the byte addresses of the non-record and record variables

in the binary file FILE, and lists of the record addresses, file

indices, and filenames for file families with history records.

*addr_lists(1)   absolute addresses of non-record variables

*addr_lists(2)   relative addresses of record variables

(add record address to get absolute address)

The order of these two address lists matches the

corresponding lists of names returned by get_vars.

*addr_lists(3)   absolute addresses of records

FROM get_addrs TO get_primitives

*addr_lists(4)   list of file indices corresponding to

addr_lists(3); indices are into addr_lists(5)

*addr_lists(5)   list of filenames in the family

SEE ALSO: openb, updateb, restore, jt, jc, has_records, get_vars

*/

get_argv

/* DOCUMENT get_argv()

returns string array containing the argv from the command line.

The -batch and batch_include.i arguments are removed (not returned).

SEE ALSO: process_argv, cd, get_cwd, get_home, get_env, batch

*/

get_cwd

/* DOCUMENT get_cwd()

or get_home()

returns the pathname of the current working directory or of your

home directory.

SEE ALSO: cd, lsdir, get_env, get_argv

*/

get_env

/* DOCUMENT get_env(environment_variable_name)

returns the environment variable (a string) associated with

ENVIRONMENT_VARIABLE_NAME (calls ANSI getenv routine).

SEE ALSO: cd, get_cwd, get_home, get_env, get_argv

*/

get_home

/* SEE get_cwd     */

get_member

/* DOCUMENT get_member(f_or_s, member_name)

returns F_OR_S member MEMBER_NAME, like F_OR_S.MEMBER_NAME syntax,

but MEMBER_NAME can be a computed string.  The F_OR_S may be a

binary file or a structure instance.

SEE ALSO: openb

*/

get_ncycs

/* SEE get_times     */

get_primitives

/* DOCUMENT prims = get_primitives(file)

Return the primitive data types for FILE as an array of 32

integers.  The format is described under set_primitives.

SEE ALSO: set_primitives, __xdr, __i86

*/

FROM get_times TO grow

get_times

/* DOCUMENT times= get_times(file)

ncycs= get_ncycs(file)

returns the list of time or ncyc values associated with the records

if FILE, or nil if there are none.  The time values are not guaranteed

to be precise (but they should be good to at least 6 digits or so);

the precise time associated with each record may be stored as a record

variable.

SEE ALSO: collect, openb, updateb, restore, jt, jc, edit_times

*/

get_vars

/* DOCUMENT name_lists= get_vars(file)

returns the lists of non-record and record variable names in the

binary FILE.  The return value is an array of two pointers to

arrays of type string; *name_lists(1) is the array of non-record

variable names (or nil if there are none), *name_lists(2) is the

array of record variable names.

The get_addrs function returns corresponding lists of disk

addresses; the get_member function can be used in conjunction

with the dimsof, structof, and typeof functions to determine

the other properties of a variable.

SEE ALSO: openb, updateb, restore, jt, jc, has_records, get_addrs,

set_vars

*/

grow

/* DOCUMENT grow, x, xnext1, xnext2, ...

or grow(x, xnext1, xnext2, ...)

or    _(x, xnext1, xnext2, ...)

lengthens the array X by appending XNEXT1, XNEXT2, etc. to its

final dimension.  If X is nil, X is first redefined to the first

non-nil XNEXT, and the remainder of the XNEXT list is processed

normally.  Each XNEXT is considered to have the same number of

dimensions as X, by appending unit-length dimensions if necessary.

All but this final dimension of each XNEXT must be right-conformable

(that is, conformable in the sense of the right hand side of an

assignment statement) with all but the final dimension of X.

The result has a final dimension which is the sum of the final

dimension of X and all the final dimensions of the XNEXT.  Nil

XNEXT are ignored.  The value of the result is obtained by

concatenating all the XNEXT to X, after any required broadcasts.

If invoked as a function, grow returns the new value of X; in

this case, X may be an expression.  X must be a simple variable

reference for the subroutine form of grow; otherwise there is

nowhere to return the result.  The subroutine form is slightly

more efficient than the function form for the common usage:

x= grow(x, xnext1, xnext2)           is the same as

grow, x, xnext1, xnext2              the preferred form

The _ function is a synonym for grow, for people who want this

operator to look like punctuation in their source code, on analogy

with the array building operator [a, b, c, ...].

FROM grow TO help

The _cat function is sometimes more appropriate than grow.

SEE ALSO: _cat, array

*/

has_records

/* DOCUMENT has_records(file)

returns 1 if FILE has history records, 0 if it does not.

*/

help

/* DOCUMENT help, topic

or help

Prints DOCUMENT comment from include file in which the variable

TOPIC was defined, followed by the line number and filename.

By opening the file with a text editor, you may be able to find

out more, especially if no DOCUMENT comment was found.

Examples:

help, set_path

prints the documentation for the set_path function.

help

prints the DOCUMENT comment you are reading.

This copy of Yorick was launched from the directory:

**** Y_LAUNCH (computed at runtime) ****

Yorick's "site directory" at this site is:

**** Y_SITE (computed at runtime) ****

You can find out a great deal more about Yorick by browsing

through these directories.  Begin with the site directory,

and pay careful attention to the subdirectories doc/ (which

contains documentation relating to Yorick), and i/ and

contrib/ (which contain many examples of Yorick programs).

Look for files called README (or something similar) in any

of these directories -- they are intended to assist browsers.

The site directory itself contains std.i and graph.i, which

are worth reading.

Type:

help, dbexit

for help on debug mode.  If your prompt is "dbug>" instead of

">", dbexit will return you to normal mode.

Type:

quit

to quit Yorick.

SEE ALSO: quit, info, print, copyright, warranty, legal

*/

FROM histogram TO include

histogram

/* DOCUMENT histogram(list)

or histogram(list, weight)

returns an array hist which counts the number of occurrences of each

element of the input index LIST, which must consist of positive

integers (1-origin index values into the result array):

histogram(list)(i) = number of occurrences of i in LIST

A second argument WEIGHT must have the same shape as LIST; the result

will be the sum of WEIGHT:

histogram(list)(i) = sum of all WEIGHT(j) where LIST(j)==i

The result of the single argument call will be of type long; the

result of the two argument call will be of type double (WEIGHT is

promoted to that type).  The input argument(s) may have any number

of dimensions; the result is always 1-D.

KEYWORD: top=max_list_value

By default, the length of the result is max(LIST).  You may

specify that the result have a larger length by means of the TOP

keyword.  (Elements beyond max(LIST) will be 0, of course.)

SEE ALSO: digitize, sort

*/

i86_primitives

/* DOCUMENT i86_primitives, file

sets FILE primitive data types to be native to Linux i86 machines.

*/

im_part

/* DOCUMENT im_part(z)

returns the imaginary part of its argument.

Unlike z.im, works if z is not complex (returns zero).

*/

include

/* DOCUMENT #include "yorick_source.i"

require, filename

include, filename

or include, filename, now

#include is a parser directive, not a Yorick statement.  Use it

to read Yorick source code which you have saved in a file; the

file yorick_source.i will be read one line at a time, exactly as

if you had typed those lines at the keyboard.  The following

directories are searched (in this order) to find yorick_source.i:

.               (current working directory)

~/Yorick        (your personal directory of Yorick functions)

Y_SITE/include  (Yorick distribution library)

Y_SITE/contrib  (contributed source at your site)

To find out what is available in the Yorick/include directory,

type:

FROM include TO install_struct

library

You can also type

Y_SITE

to find the name of the site directory at your site, go to the

include or contrib subdirectory, and browse through the *.i files.

This is a good way to learn how to write a Yorick program.  Be

alert for files like README as well.

The require function checks to see whether FILENAME has already

been included (actually whether any file with the same final

path component has been included).  If so, require is a no-op,

otherwise, the action is the same as the include function with

NOW == 1.

The include function causes Yorick to parse and execute FILENAME

immediately.  The effect is similar to the #include parser

directive, except the finding, parsing, and execution of FILENAME

occurs at runtime.  If the NOW argument is given and positive,

the include occurs immediately, if nil or 0, it occurs just before

the next line would have been parsed.  If NOW is negative, the

include file is pushed onto a stack, and will be popped off and

parsed when all pending input has been processed.

Unless you are writing a startup file, or have some truly bizarre

technical reason for using the include function, use #include

instead.  The functional form of include may involve recursive

parsing, which you will not be able to understand without deep

study.  Stick with #include.

SEE ALSO: set_path, Y_SITE

*/

indgen

/* DOCUMENT indgen(n)

or indgen(start:stop)

or indgen(start:stop:step)

returns "index generator" list -- an array of longs running from

1 to N, inclusive.  In the second and third forms, the index

values specified by the index range are returned.

SEE ALSO: span, spanl, array

*/

info

/* DOCUMENT info, expr

prints the data type and array dimensions of EXPR.

SEE ALSO: help, print

*/

install_struct

/* DOCUMENT install_struct, file, struct_name

or install_struct, file, struct_name, size, align, order

or install_struct, file, struct_name, size, align, order, layout

installs the data type named STRUCT_NAME in the binary FILE.  In

the two argument form, STRUCT_NAME must have been built by one or

more calls to the add_member function.  In the 5 and 6 argument calls,

FROM install_struct TO interp

STRUCT_NAME is a primitive data type -- an integer type for the 5

argument call, and a floating point type for the 6 argument call.

The 5 argument form may also be used to declare opaque data types.

SIZE is the size of an instance in bytes, ALIGN is its alignment

boundary (also in bytes), and ORDER is the byte order.  ORDER is

1 for most significant byte first, -1 for least significant byte

first, and 0 for opaque (unconverted) data.  Other ORDER values

represent more complex byte permutations (2 is the byte order for

VAX floating point numbers).  If ORDER equals SIZE, then the data

type is not only opaque, but also must be read sequentially.

LAYOUT is an array of 7 long values parameterizing the floating

point format, [sign_address, exponent_address, exponent_size,

mantissa_address, mantissa_size, mantissa_normalized, exponent_bias]

(the addresses and sizes are in bits, reduced to MSB first order).

Use, e.g., nameof(float) for STRUCT_NAME to redefine the meaning

of the float data type for FILE.

SEE ALSO: add_variable, add_member

*/

integ

/* DOCUMENT integ(y, x, xp)

or integ(y, x, xp, which)

See the interp function for an explanation of the meanings of the

arguments.  The integ function returns ypi which is the integral

of the piecewise linear curve (X(i), Y(i)) (i=1, ..., numberof(X))

from X(1) to XP.  The curve (X, Y) is regarded as constant outside

the bounds of X.  Note that X must be monotonically increasing or

SEE ALSO: interp, digitize, span

*/

interp

/* DOCUMENT interp(y, x, xp)

or interp(y, x, xp, which)

returns yp such that (XP, yp) lies on the piecewise linear curve

(X(i), Y(i)) (i=1, ..., numberof(X)).  Points beyond X(1) are set

to Y(1); points beyond X(0) are set to Y(0).  The array X must be

one dimensional, have numberof(X)>=2, and be either monotonically

increasing or monotonically decreasing.  The array Y may have more

than one dimension, but dimension WHICH must be the same length as

X.  WHICH defaults to 1, the first dimension of Y.  WHICH may be

non-positive to count dimensions from the end of Y; a WHICH of 0

means the final dimension of Y.  The result yp has dimsof(XP)

in place of the WHICH dimension of Y (if XP is scalar, the WHICH

dimension is not present).  (The dimensions of the result are the

same as if an index list with dimsof(XP) were placed in slot

WHICH of Y.)

SEE ALSO: integ, digitize, span

*/

FROM is_array TO jc

is_array

/* DOCUMENT is_array(object)

returns 1 if OBJECT is an array data type (as opposed to a function,

structure definition, index range, I/O stream, etc.), else 0.

An array OBJECT can be written to or read from a binary file;

non-array Yorick data types cannot.

SEE ALSO: is_func, is_void, is_range, is_struct, is_stream

*/

is_func

/* DOCUMENT is_func(object)

returns 1 if OBJECT is a Yorick interpreted function, 2 if OBJECT

is a built-in (that is, compiled) function, else 0.

SEE ALSO: is_array, is_void, is_range, is_struct, is_stream

*/

is_range

/* DOCUMENT is_range(object)

returns 1 if OBJECT is an index range (e.g.-  3:5 or 11:31:2),

else 0.

SEE ALSO: is_array, is_func, is_void, is_struct, is_stream

*/

is_stream

/* DOCUMENT is_stream(object)

returns 1 if OBJECT is a binary I/O stream (usually a file), else 0.

The _read and _write functions work on object if and only if

is_stream returns non-zero.  Note that is_stream returns 0 for a

text stream -- you need the typeof function to test for those.

SEE ALSO: is_array, is_func, is_void, is_range, is_struct

*/

is_struct

/* DOCUMENT is_struct(object)

returns 1 if OBJECT is the definition of a Yorick struct, else 0.

Thus, is_struct(double) returns 1, but is_struct(1.0) returns 0.

SEE ALSO: is_array, is_func, is_void, is_range, is_stream

*/

is_void

/* DOCUMENT is_void(object)

returns 1 if OBJECT is nil (the one instance of the void data type),

else 0.

SEE ALSO: is_array, is_func, is_range, is_struct, is_stream

*/

jc

/* DOCUMENT jc, file, ncyc

jump to the record of FILE nearest the specified NCYC.

SEE ALSO: jt, _jc, edit_times, show, jr

*/

FROM jr TO legal

jr

/* DOCUMENT jr, file, i

or _jr(file, i)

Jump to a particular record number I (from 1 to n_records) in a

binary file FILE.  The function returns 1 if such a record exists,

0 if there is no such record.  In the latter case, no action is

taken; the program halts with an error only if jr was invoked

as a subroutine.  Record numbering wraps like array indices; use

jr, file, 0  to jump to the last record, -1 to next to last, etc.

SEE ALSO: jt, jc, edit_times, show

*/

jt

/* DOCUMENT jt, time

or jt, file, time

or jt, file

or jt, file, -

jump to the record nearest the specified TIME.  If no FILE is

specified, the current record of all open binary files containing

records is shifted.

If both FILE and TIME are specified and jt is called as a function,

it returns the actual time of the new current record.

N.B.: "jt, file" and "jt, file, -" are obsolete.  Use the jr function to

step through a file one record at a time.

If only the FILE is specified, increment the current record of that

FILE by one.  If the TIME argument is - (the pseudo-index range

function), decrement the current record of FILE by one.

If the current record is the last, "jt, file" unsets the current record

so that record variables will be inaccessible until another jt or jc.

The same thing happens with "jt, file, -" if the current record was the

first.

If only FILE is specified, jt returns 1 if there is a new current

record, 0 if the call resulted in no current record.  Thus "jt(file)"

and "jt(file,-)" may be used as the condition in a while loop to step

through every record in a file:

file= openb("example.pdb");

do {

restore, file, interesting_record_variables;

...calculations...

} while (jt(file));

SEE ALSO: jc, _jt, edit_times, show, jr

*/

legal

/* DOCUMENT legal

Prints the legal details of Yorick's copyright, licensing,

and lack of warranty.

SEE ALSO: copyright, warranty

*/

FROM library TO max

library

/* DOCUMENT library

print the Y_SITE/i/README file at the terminal.

*/

log

/* DOCUMENT log(x)

returns the natural logarithm of its argument (inverse of exp).

SEE ALSO: log10, exp, asinh, acosh, atanh

*/

log10

/* DOCUMENT log10(x)

returns the base 10 logarithm of its argument (inverse of 10^x).

SEE ALSO: log, exp, asinh, acosh, atanh

*/

lsdir

/* DOCUMENT files = lsdir(directory_name)

or files = lsdir(directory_name, subdirs)

List DIRECTORY_NAME.  The return value FILES is an array of

strings or nil; the order of the filenames is unspecified;

it does not contain "." or ".."; it does not contain the

names of subdirectories.  If SUBDIRS is given and is a simple

variable name, it is set to a list of subdirecotry names (or

nil if there are no subdirectories).

If DIRECTORY_NAME does not exist, the return value is the

integer 0 rather than nil.

SEE ALSO: cd, mkdir, rmdir, get_cwd, get_home

*/

mac_primitives

/* DOCUMENT mac_primitives, file

sets FILE primitive data types to be native to MacIntosh, 8 byte double.

*/

macl_primitives

/* DOCUMENT macl_primitives, file

sets FILE primitive data types to be native to MacIntosh, long double.

*/

max

/* DOCUMENT max(x)

or max(x, y, z, ...)

returns the scalar maximum value of its array argument, or, if

more than one argument is supplied, returns an array of the

maximum value for each array element among the several arguments.

In the multi-argument case, the arguments must be conformable.

SEE ALSO: min, sum, avg

*/

FROM median TO merge

median

/* DOCUMENT median(x)

or median(x, which)

returns the median of the array X.  The search for the median takes

place along the dimension of X specified by WHICH.  WHICH defaults

to 1, meaning the first index of X.  The median function returns an

array with one fewer dimension than its argument X (the WHICH

dimension of X is missing in the result), in exact analogy with

rank reducing index range functions.  If dimsof(X)(WHICH) is

odd, the result will have the same data type as X; if even, the

result will be a float or a double, since the median is defined

as the arithmetic mean between the two central values in that

case.

SEE ALSO: sort

*/

merge

/* DOCUMENT merge(true_expr, false_expr, condition)

returns the values TRUE_EXPR or FALSE_EXPR where CONDITION is

non-zero or zero, respectively.  The result has the data type of

TRUE_EXPR or FALSE_EXPR, promoted to the higher arithmetic type

if necessary.  The result has the dimensions of CONDITION.

The number of elements in TRUE_EXPR must match the number of

non-zero elements of CONDITION, and the number of elements in

FALSE_EXPR must match the number of zero elements of CONDITION.

(TRUE_EXPR or FALSE_EXPR should be nil if there are no such

elements of CONDITION.  Normally, TRUE_EXPR and FALSE_EXPR should

be 1-D arrays if they are not nil.)

This function is intended for vectorizing a function whose

domain is divided into two or more parts, as in:

func f(x) {

big= (x>=threshhold);

wb= where(big);

ws= where(!big);

if (is_array(wb)) {

xx= x(wb);

fb= <function of xx>

}

if (is_array(ws)) {

xx= x(ws);

fs= <function of xx>

}

return merge(fb, fs, big);

}

SEE ALSO: merge2, where

*/

FROM merge2 TO open

merge2

/* DOCUMENT merge2(true_expr, false_expr, condition)

returns the values TRUE_EXPR or FALSE_EXPR where CONDITION is

non-zero or zero, respectively.  The result has the data type of

TRUE_EXPR or FALSE_EXPR, promoted to the higher arithmetic type

if necessary.  Unlike the merge function, TRUE_EXPR and FALSE_EXPR

must be conformable with each other, and with the CONDITION.

SEE ALSO: merge2, where

*/

min

/* DOCUMENT min(x)

or min(x, y, z, ...)

returns the scalar minimum value of its array argument, or, if

more than one argument is supplied, returns an array of the

minimum value for each array element among the several arguments.

In the multi-argument case, the arguments must be conformable.

SEE ALSO: max, sum, avg

*/

mkdir

/* DOCUMENT mkdir, directory_name

rmdir, directory_name

Create DIRECTORY_NAME with mkdir, or remove it with rmdir.

The rmdir function only works if the directory is empty.

SEE ALSO: cd, lsdir, get_cwd, get_home

*/

nallof

/* SEE allof     */

nameof

/* DOCUMENT nameof(object)

If OBJECT is a function or a structure definition, returns the

name of the func or struct as it was defined (not necessarily

the name of the variable passed to the nameof function).

SEE ALSO: typeof

*/

noneof

/* SEE allof     */

numberof

/* DOCUMENT numberof(object)

returns the number of elements if object is an array, or 0 if not.

SEE ALSO: sizeof, dimsof, typeof, structof

*/

open

/* DOCUMENT f= open(filename)

or f= open(filename, filemode)

or f= open(filename, filemode, errmode)

opens the file FILENAME according to FILEMODE (both are strings).

If ERRMODE is non-nil and non-zero, fail by returning nil F,

otherwise failure to open or create the file is a runtime error.

FROM open TO openb

To use ERRMODE to check for the existence of a file:

if (open(filename,"r",1)) file_exists;

else file_does_not_exist;

The return value F is an IOStream (or just stream for short).  When

the last reference to this return value is discarded, the file will

be closed.  The file can also be explicitly closed with the close

function.  The FILEMODE determines whether the file is to be

opened in read, write, or update mode, and whether writes are

restricted to the end-of-file (append mode).  FILEMODE also

determines whether the file is opened as a text file or as a

binary file.  FILEMODE can have the following values, which are

the same as for the ANSI standard fopen function:

"r"     - read only

"w"     - write only, random access, existing file overwritten

"a"     - write only, forced to end-of-file,

existing file preserved

"r+"    - read/write, random access, existing file preserved

"w+"    - read/write, random access, existing file overwritten

"a+"    - read/write, reads random access,

writes forced to end-of-file, existing file preserved

"rb"  "wb"  "ab"  "r+b"  "rb+"  "w+b"  "wb+"  "a+b"  "ab+"

without b means text file, with b means binary file

The default FILEMODE is "r" -- open an existing text file for

reading.

The read and write functions perform I/O on text files.

I/O to binary files may be performed explicitly using the save

and restore functions, or implicitly by using the stream variable

F as if it were a data structure instance (e.g.- f.x refers to

variable x in the binary file f).

SEE ALSO: create, close, read, write, rdline, bookmark, backup, popen

rename, remove, save, restore

*/

open102

/* SEE close102     */

openb

/* DOCUMENT file= openb(filename)

or file= openb(filename, clogfile)

open the existing file FILENAME for read-only binary I/O.

(Use updateb or createb, respectively, to open an existing file

with read-write access or to create a new file.)

If the CLOGFILE argument is supplied, it represents the structure

of FILENAME in the Clog binary data description language.

After an openb, the file variable may be used to extract variables

from the file as if it were a structure instance.  That is, the

expression "file.var" refers to the variable "var" in file "file".

A complete list of the variable names present in the file may

be obtained using the get_vars function.  If the file contains

history records, the jt and jc functions may be used to set the

current record -- initially, the first record is current.

The restore function may be used to make memory copies of data

FROM openb TO poly

in the file; this will be faster than a large number of

references to "file.var".

SEE ALSO: updateb, createb, open, cd

show, jt, jc, restore

get_vars, get_times, get_ncycs, get_member, has_records

set_blocksize, dump_clog, read_clog, recover_file

openb_hooks, open102, close102, get_addrs

*/

openb_hooks

/* DOCUMENT openb_hooks

list of functions to be tried by openb if the file to be

opened is not a PDB file.  By default,

openb_hooks= _lst(_not_pdbf, _not_cdf).

The hook functions will be called with the file as argument

(e.g.- _not_cdf(file)), beginning with _car(openb_hooks), until

one of them returns 0.  Note that a hook should return 0 if it

"recognizes" the file as one that it should be able to open, but

finds that the file is misformatted (alternatively, it could call

error to abort the whole process).

*/

orgsof

/* DOCUMENT orgsof(object)

returns a vector of integers describing the dimensions of OBJECT.

The format of the vector is [number of dims, origin1, origin2, ...].

By default, dimension origins are ignored, but use_origins changes

this.  The dimsof function returns the length of each dimension.

*** NOTE NOTE NOTE ***

Unless use_origins(1) is in effect, orgsof will always return

1 for all of the originI in the list.  Thus, whether use_origins(1)

is in effect or not, you are guaranteed that x(orgsof(x)(2)) is the

first element of x.

SEE ALSO: dimsof, typeof, structof, numberof, sizeof, use_origins

*/

pc_primitives

/* DOCUMENT pc_primitives, file

sets FILE primitive data types to be native to IBM PC.

*/

pi

/* DOCUMENT pi

roughly 3.14159265358979323846264338327950288

*/

poly

/* DOCUMENT poly(x, a0, a1, a2, ..., aN)

returns the polynomial  A0 + A1*x + A2*x^2 + ... + AN*X^N

The data type and dimensions of the result, and conformability rules

for the inputs are identical to those for the expression.

*/

FROM popen TO process_argv

popen

/* DOCUMENT f= popen(command, mode)

opens a pipe to COMMAND, which is executed as with the system

function.  If MODE is 0, the returned file handle is open for

reading, and you are reading the stdout produced by COMMAND.

If MODE is 1, f is opened for writing and you are writing to

the stdin read by COMMAND.

SEE ALSO: open, system

*/

pr1

/* DOCUMENT pr1(x)

returns text representing expression X, equivalent to print(X)(1).

SEE ALSO: print, swrite

*/

print

/* DOCUMENT print, object1, object2, object3, ...

or print(object1, object2, object3, ...)

prints an ASCII representation of the OBJECTs, in roughly the format

they could appear in Yorick source code.  When invoked as a subroutine

(in the first form), output is to the terminal.  When invoked as a

function (int the second form), the output is stored as a vector of

strings, one string per line that would have been output.

Printing a structure definition prints the structure definition;

printing a function prints its "func" definition; printing files,

bookmarks, and other objects generally provides some sort of

useful description of the object.

SEE ALSO: pr1, print_format, write, exit, error, nameof, typeof

*/

print_format

/* DOCUMENT print_format, line_length, char=, short=, int=, float=,

double=, complex=, pointer=

sets the format string the print function will use for each of

the basic data types.  Yorick format strings are the same as the

format strings for the printf function defined in the ANSI C standard.

The default strings may be restored individually by setting the

associated format string to ""; all defaults are restored if

print_format is invoked with no arguments.  The default format strings

are:  "0x%02x", "%d", "%d", "%ld", "%g", "%g", and "%g%+gi".

Note that char and short values are converted to int before being

passed to printf, and that float is converted to double.

If present, an integer positional argument is taken as the line

length; <=0 restores the default line length of 80 characters.

SEE ALSO: print, write, nameof, typeof

*/

process_argv

/* DOCUMENT remaining= process_argv()

-or- remaining= process_argv("your startup message")

Performs standard command line processing.  This function is

invoked by the default custom.i file (in $Y_HOME/include); you

can also invoke it from your personal ~/Yorick/custom.i file.

The process_argv calls get_argv, removes any arguments of

FROM process_argv TO random_seed

the form "-ifilename" or "-i filename" (the latter is a pair of

arguments.  It returns any arguments not of this form as its

result, after including any filenames it found in the order

they appeared on the command line.

The optional string argument may be an array of strings to print

a multi-line message.

A Yorick package may define the function get_command_line in

order to feed process_argv something other than get_argv.

SEE ALSO: batch

*/

ptcen

/* DOCUMENT ptcen(zncen)

or ptcen(zncen, ireg)

returns point centered version of the 2-D zone centered array ZNCEN.

The result is imax-by-jmax if ZNCEN is (imax-1)-by-(jmax-1).

If the region number array IREG is specified, zones with region

number 0 are not included in the point centering operation.

Note that IREG should have dimensions imax-by-jmax; the first

row and column of IREG are ignored.

Without IREG, ptcen(zncen) is equivalent to zncen(pcen,pcen).

SEE ALSO: zncen, uncen

*/

quit

/* DOCUMENT quit

Exit YMainLoop when current task finishes.

Normally this terminates the program.

*/

random

/* DOCUMENT random(dimension_list)

random_seed, seed

returns an array of random double values with the given

DIMENSION_LIST (nil for a scalar result), uniformly distributed

on the interval from 0.0 to 1.0.

The algorithm is from Press and Teukolsky, Computers in Physics,

vol. 6, no. 5, Sep/Oct 1992 (ran2).  They offer a reward of $1000

to anyone who can exhibit a statistical test that this random

number generator fails in a "non-trivial" way.

The random_seed call reinitializes the random number sequence;

SEED should be between 0.0 and 1.0 non-inclusive; if SEED is

omitted, nil, or out of range, the sequence is reinitialized as

when Yorick starts.

The numbers are actually at the centers of 2147483562 equal width

bins on the interval [0,1].  Although only these 2 billion numbers

are possible, the period of the generator is roughly 2.3e18.

SEE ALSO: randomize

*/

random_seed

/* SEE random     */

FROM randomize TO read

randomize

/* DOCUMENT randomize

randomize()

set the seed for random "randomly" (based on the timer clock

and the current state of random).  As a function, returns the

value of the seed passed to random_seed.

SEE ALSO: random, random_seed

*/

rdline

/* DOCUMENT rdline(f)

or rdline(f, n, prompt= pstring)

returns next line from stream F (stdin if F nil).  If N is non-nil,

returns a string array containing the next N lines of F.  If

end-of-file occurs, rdline returns nil strings.  If F is nil,

uses the PSTRING to prompt for input (default "read> ").

SEE ALSO: read, open, close, bookmark, backup, read_n

*/

re_part

/* DOCUMENT re_part(z)

returns the real part of its argument.  (Same as double(z).)

Unlike z.re, works if z is not complex.

*/

read

/* DOCUMENT n= read(f, format=fstring, obj1, obj2, ...)

or n= read(prompt= pstring, format=fstring, obj1, obj2, ...)

or n= sread(source, format=fstring, obj1, obj2, ...)

reads text from I/O stream F (1st form), or from the keyboard (2nd

form), or from the string or string array SOURCE (3rd form),

interprets it according to the optional FSTRING, and uses that

interpretation to assign values to OBJ1, OBJ2, ...  If the input

is taken from the keyboard, the optional prompt PSTRING (default

"read> ") is printed before each line is read.  The Yorick write

function does not interact with the read function -- writes are

always to end-of-file, and do not affect the sequence of lines

returned by read.  The backup (and bookmark) function is the

only way to change the sequence of lines returned by read.

There must be one non-supressed conversion specifier (see below)

in FSTRING for each OBJ to be read; the type of the conversion

specifier must generally match the type of the OBJ.  That is,

an integer OBJ requires an integer specifier (d, i, o, u, or x)

in FSTRING, a real OBJ requires a real specifier (e, f, or g),

and a string OBJ requires a string specifier (s or []).  An OBJ

may not be complex, a pointer, a structure instance, or any non-

array Yorick object.  If FSTRING is not supplied, or if it has

fewer conversion specifiers than the number of OBJ arguments,

then Yorick supplies default specifiers ("%ld" for integers,

"%lg" for reals, and "%s" for strings).  If FSTRING contains more

specifiers than there are OBJ arguments, the part of FSTRING

beginning with the first specifier with no OBJ is ignored.

FROM read TO read

The OBJ may be scalar or arrays, but the dimensions of every OBJ

must be identical.  If the OBJ are arrays, Yorick behaves as

if the read were called in a loop numberof(OBJ1) times, filling

one array element of each of the OBJ according to FSTRING on

each pass through the loop.  (Note that this behavior includes

the case of reading columns of numbers by a single call to read.)

The return value N is the total number of scalar assignments

which were made as a result of this call.  (If there were 4

OBJ arguments, and each was an array with 17 elements, a return

value of N==35 would mean the following:  The first 8 elements

of OBJ1, OBJ2, OBJ3, and OBJ4 were read, and the 9th element of

OBJ1, OBJ2, and OBJ3 was read.)  The read function sets any

elements of the OBJ which were not read to zero -- hence,

independent of the returned N, the all of the old data in the

OBJ arguments is overwritten.

The read or sread functions continue reading until either:

(1) all elements of all OBJ have been filled, or (2) end-of-file

(or end of SOURCE for sread) is reached ("input failure"), or

(3) part of FSTRING or a conversion specifier supplied by

default fails to match the source text ("matching failure").

The FSTRING is composed of a series of "directives" which are

(1) whitespace -- means to skip any amount of whitespace in the

source text

(2) characters other than whitespace and % -- must match the

characters in the source text exactly, or matching failure

occurs and the read operation stops

(3) conversion specifiers beginning with % and ending with a

character specifying the type of conversion -- optionally

skip whitespace, then convert as many characters as

continue to "look like" the conversion type, possibly

producing a matching failure

The conversion specifier is of the form %*WSC, where:

* is either the character '*' or not present

A specifier beginning with %* does not correspond to any of

the OBJ; the converted value will be discarded.

W is either a positive decimal integer specifying the maximum

field width (not including any skipped leading whitespace),

or not present if any number of characters up to end-of-line

is acceptable.

S is either one of the characters 'h', 'l', or 'L', or not

present.  Yorick allows this for compatibility with the C

library functions, but ignores it.

C is a character specifying the type of conversion:

d   - decimal integer

i   - decimal, octal (leading 0), or hex (leading 0x) integer

o   - octal integer

u   - unsigned decimal integer (same as d for Yorick)

x, X            - hex integer

e, f, g, E, G   - floating point real

s   - string of non-whitespace characters

[xxx]   - (xxx is any sequence of characters) longest string

of characters matching those in the list

FROM read TO rename

[^xxx]  - longest string of characters NOT matching those in

the list (this is how you can extend %s to be

delimited by something other than whitespace)

%   - the ordinary % character; complete conversion

specification must be "%%"

The read function is modeled on the ANSI standard C library

fscanf and sscanf functions, but differs in several respects:

(1) Yorick's read cannot handle the %c, %p, or %n conversion

specifiers in FSTRING.

(2) Yorick's read never results in a portion of a line

being read -- any unused part of a line is simply discarded

(end FSTRING with "%[^\n]" if you want to save the trailing

part of an input line).

(3) As a side effect of (2), there are some differences between

fscanf and Yorick's read in how whitespace extending across

newlines is handled.

SEE ALSO: rdline, write, open, close, bookmark, backup, save, restore,

read_n

*/

read_clog

/* DOCUMENT file= read_clog(file, clog_name)

raw routine to set the binary data structure of FILE according

to the text description in the Contents Log file CLOG_NAME.

*/

read_n

/* DOCUMENT read_n, f, n0, n1, n2, ...

grabs the next numbers N0, N1, N2, ... from file F, skipping over

any whitespace, comma, semicolon, or colon delimited tokens which

are not numbers.  (Actually, only the first and last characters of

the token have to look like a number -- 4xxx3 would be read as 4.)

***WARNING*** at most ten Ns are allowed

The Ns can be arrays, provided all have the same dimensions.

SEE ALSO: read, rdline

*/

recover_file

/* DOCUMENT recover_file, filename

or recover_file, filename, clogfile

writes the descriptive information at the end of a corrupted

binary file FILENAME from its Contents Log file CLOGFILE, which

is FILENAME+"L" by default.

*/

remove

/* SEE rename     */

rename

/* DOCUMENT rename, old_filename, new_filename

remove filename

rename or remove a file.

SEE ALSO: open, close, openb

*/

FROM require TO save

require

/* SEE include     */

reshape

/* DOCUMENT reshape, reference, address, type, dimension_list

or reshape, reference, type, dimension_list

or reshape, reference

The REFERENCE must be an unadorned variable, not an expression;

reshape sets this variable to an LValue at the specified ADDRESS

with the specified TYPE and DIMENSION_LIST.  (See the array

function documentation for acceptable DIMENSION_LIST formats.)

If ADDRESS is an integer (e.g.- a long), the programmer is

responsible for assuring that the data at ADDRESS is valid.

If ADDRESS is a (Yorick) pointer, Yorick will assure that the

data pointed to will not be discarded, and the reshape will

fail if TYPE and DIMENSION_LIST extend beyond the pointee

bounds.  In the second form, ADDRESS is taken to be &REFERENCE;

that is, the TYPE and DIMENSION_LIST of the variable are changed

without doing any type conversion.  In the third form, REFERENCE

is set to nil ([]).  (Simple redefinition will not work on a

variable defined using reshape.)

WARNING: There are almost no situations for which reshape is

the correct operation.  See reform in Y_SITE/i/string.i.

SEE ALSO: array, dimsof, numberof, is_array, eq_nocopy

*/

restore

/* SEE save     */

rmdir

/* SEE mkdir     */

save

/* DOCUMENT save, file, var1, var2, ...

restore, file, var1, var2, ...

saves the variables VAR1, VAR2, etc. in the binary file FILE,

or restores them from that file.

The VARi may be either non-record or record data in the case that

FILE contains records.

If one of the VARi does not already exist in FILE, it is created

by the save command; after add_record, save adds or stores VARi to

the current record.  See add_record for more.  The VARi may be

structure definitions (for the save command) to declare data

structures for the file.  This is necessary only in the case that

a record variable is a pointer -- all of the potential data types

of pointees must be known.  No data structures may be declared

using the save command after the first record has been added.

If no VARi are present, save saves all array variables, and

restore restores every non-record variable in the file if there

FROM save TO set_path

is no current record, and every variable in the current record if

there is one.

SEE ALSO: openb, createb, updateb, get_vars, add_record, get_addrs

jt, jc, _read, _write, data_align

*/

sech

/* DOCUMENT sech(x)

csch(x)

returns the hyperbolic secant (1/cosh) or cosecant (1/sinh) of

its argument, without overflowing for large x.

SEE ALSO: sinh, cosh, tanh, asinh, acosh, atanh

*/

set_blocksize

/* DOCUMENT set_blocksize, file, blocksize

sets smallest cache block size for FILE to BLOCKSIZE.  BLOCKSIZE

is rounded to the next larger number of the form 4096*2^n if

necessary; cache blocks for this file will be multiples of

BLOCKSIZE bytes long.  The default BLOCKSIZE is 0x4000 (16 KB).

SEE ALSO: openb, updateb, createb, save, restore, _read, _write

*/

set_filesize

/* DOCUMENT set_filesize, file, filesize

sets the new family member threshhold for FILE to FILESIZE.

Whenever a new record is added (see add_record), if the current file

in the FILE family has at least one record and the new record would

cause the current file to exceed FILESIZE bytes, a new family

member will be created to hold the new record.

The default FILESIZE is 0x400000 (4 MB).

SEE ALSO: openb, updateb, createb, add_record

*/

set_idler

/* DOCUMENT set_idler, idler_function

sets the idler function to IDLER_FUNCTION.  Instead of waiting

for keyboard input when all its tasks are finished, the interpreter

will invoke IDLER_FUNCTION with no arguments.  The idler function

is normally invoked only once, so input from the keyboard resumes

after one call to the idler.  Of course, an idler is free to call

set_idler again before it returns, which will have the effect of

calling that function in a loop.

SEE ALSO: batch

*/

set_path

/* DOCUMENT set_path, "dir1:dir2:dir3:..."

or set_path

sets the include file search path to the specified list of

directories.  The specified directories are searched left to

right for include files specified as relative file names in

#include directives, or to the include or require functions.

If the argument is omitted, restores the default search path,

".:~/Yorick:Y_LAUNCH/include:Y_SITE/include:Y_SITE/contrib",

FROM set_path TO sgi64_primitives

where y_site is the main Yorick directory for this site.

The Y_LAUNCH directory is the directory which contains the

executable; this directory is omitted if it is the same as

Y_SITE.

Only the "end user" should ever call set_path, and then only in

his or her custom.i file, for the purpose of placing a more

elaborate set of personal directories containing Yorick procedures.

For example, if someone else maintains Yorick code you use, you

might put their ~/Yorick on your include path.

SEE ALSO: Y_LAUNCH, Y_SITE, include, require

*/

set_primitives

/* DOCUMENT set_primitives, file, prims

Return the primitive data types for FILE as an array of 32

integers.  Versions for particular machines are defined in

prmtyp.i, and can be accessed using functions like

sun_primitives or i86_primitives.  See __xdr for a complete

list.  The format is:

[size, align, order] repeated 6 times for char, short, int,

long, float, and double, except that char align is always 1,

so result(2) is the structure alignment (see struct_align).

[sign_address,  exponent_address, exponent_bits,

mantissa_address, mantissa_bits,

mantissa_normalization, exponent_bias] repeated twice for

float and double.  See the comment at the top of prmtyp.i

for an explanation of these fields.

the total number of items is thus 3*6+7*2=32.

SEE ALSO: get_primitives, createb, __xdr, __i86

*/

set_vars

/* DOCUMENT set_vars, file, names

or set_vars, file, nonrec_names, rec_names

Change the names of the variables in FILE to NAMES.  If the

file has record variables, you can use the second form to change

the record variable names.  Either of the two lists may be nil

to leave those names unchanged, but if either is not nil, it must

be a 1D array of strings whose length exactly matches the number

of that type of variable actually present in the file.

SEE ALSO: openb, updateb, has_records, get_vars

*/

sgi64_primitives

/* DOCUMENT sgi64_primitives, file

sets FILE primitive data types to be native to 64-bit SGI workstations.

*/

FROM show TO sort

show

/* DOCUMENT show, f

or show, f, pat

or show, f, 1

prints a summary of the variables contained in binary file F.

If there are too many variables, use the second form to select

only those variables whose first few characters match PAT.

In the third form, continues the previous show command where it

left off -- this may be necessary for files with large numbers of

variables.

The variables are printed in alphabetical order down the columns.

The print function can be used to obtain other information about F.

SEE ALSO: openb, jt, jc

*/

sign

/* DOCUMENT sign(x)

returns algebraic sign of it argument, or closest point on the

unit circle for complex x.  Guaranteed that x==sign(x)*abs(x).

sign(0)==+1.

SEE ALSO: abs

*/

sin

/* DOCUMENT sin(x)

cos(x)

tan(x)

returns the sine, cosine, or tangent of its argument,

which is in radians.

SEE ALSO: asin, acos, atan

*/

sinh

/* DOCUMENT sinh(x)

cosh(x)

tanh(x)

returns the hyperbolic sine, cosine, or tangent of its argument.

SEE ALSO: sech, csch, asinh, acosh, atanh

*/

sizeof

/* DOCUMENT sizeof(object)

returns the size of the object in bytes, or 0 for non-array objects.

sizeof(structure_definition) returns the number of bytes per instance.

sizeof(binary_file) returns the file size in bytes.

SEE ALSO: dimsof, typeof, structof, numberof

*/

sort

/* DOCUMENT sort(x)

or sort(x, which)

returns an array of longs with dimsof(X) containing index values

such that X(sort(X)) is a monotonically increasing array.  X can

contain integer, real, or string values.  If X has more than one

dimension, WHICH determines the dimension to be sorted.  The

FROM sort TO sread

default WHICH is 1, corresponding to the first dimension of X.

WHICH can be non-positive to count dimensions from the end of X;

in particular a WHICH of 0 will sort the final dimension of X.

WARNING: The sort function is non-deterministic if some of the

values of X are equal, because the Quick Sort algorithm

involves a random selection of a partition element.

For information on sorting with multiple keys (and on making

sort deterministic), type the following:

#include "msort.i"

help, msort

SEE ALSO: median, digitize, interp, integ, histogram

*/

span

/* DOCUMENT span(start, stop, n)

or span(start, stop, n, which)

returns array of N doubles equally spaced from START to STOP.

The START and STOP arguments may themselves be arrays, as long as

they are conformable.  In this case, the result will have one

dimension of length N in addition to dimsof(START, STOP).

By default, the result will be N-by-dimsof(START, STOP), but

if WHICH is specified, the new one of length N will be the

WHICHth.  WHICH may be non-positive to position the new

dimension relative to the end of dimsof(START, STOP); in

particular WHICH of 0 produces a result with dimensions

dimsof(START, STOP)-by-N.

SEE ALSO: spanl, indgen, array

*/

spanl

/* DOCUMENT spanl(start, stop, n)

or spanl(start, stop, n, which)

similar to the span function, but the result array have N points

spaced at equal ratios from START to STOP (that is, equally

spaced logarithmically).  See span for discussion of WHICH argument.

START and STOP must have the same algebraic sign for this to make

any sense.

SEE ALSO: span, indgen, array

*/

sqrt

/* DOCUMENT sqrt(x)

returns the square root of its argument.

SEE ALSO: abs

*/

sread

/* SEE read     */

FROM strlen TO strtok

strlen

/* DOCUMENT strlen(string_array)

returns an long array with dimsof(STRING_ARRAY) containing the

lengths of the strings.  The null string (0) is considered to

have length 0, just like "".

SEE ALSO: strmatch, strpart, strtok

*/

strmatch

/* DOCUMENT strmatch(string_array, pattern)

or strmatch(string_array, pattern, case_fold)

returns an int array with dimsof(STRING_ARRAY) with 0 where

PATTERN was not found in STRING_ARRAY and 1 where it was found.

If CASE_FOLD is specified and non-0, the pattern match is

insensitive to case, that is, an upper case letter will match

the same lower case letter and vice-versa.

SEE ALSO: strtok, strpart, strlen

*/

strpart

/* DOCUMENT strpart(string_array, m:n)

returns another string array with the same dimensions as

STRING_ARRAY which consists of characters M through N of

the original strings.  M and N are 1-origin indices; if

M is omitted, the default is 1; if N is omitted, the default

is the end of the string.  If M or N is non-positive, it is

interpreted as an index relative to the end of the string,

with 0 being the last character, -1 next to last, etc.

Finally, the returned string will be shorter than N-M+1

characters if the original doesn't have an Mth or Nth

character, with "" (note that this is otherwise impossible)

if neither an Mth nor an Nth character exists.  A 0

is returned for any string which was 0 on input.

SEE ALSO: strmatch, strtok, strlen

*/

strtok

/* DOCUMENT strtok(string_array, delimiters)

or strtok(string_array)

strips the first token off of each string in STRING_ARRAY.

A token is delimited by any of the characters in the string

DELIMITERS.  If DELIMITERS is blank, nil, or not given, the

default DELIMITERS is " \t\n" (blanks, tabs, or newlines).

The result is a string array ts with dimensions

2-by-dimsof(STRING_ARRAY); ts(1,) is the first token, and

ts(2,) is the remainder of the string (the character which

terminated the first token will be in neither of these parts).

The ts(2,) part will be 0 (i.e.- the null string) if no more

characters remain after ts(1,); the ts(1,) part will be 0 if

no token was present.  A STRING_ARRAY element may be 0, in

which case (0, 0) is returned for that element.

SEE ALSO: strmatch, strpart, strlen

*/

FROM struct_align TO symbol_def

struct_align

/* DOCUMENT struct_align, file, alignment

in binary file FILE, align new struct members which are themselves

struct instances to begin at a byte address which is a multiple of

ALIGNMENT.  (This affects members declared explicitly by add_member,

as well as implicitly by save or add_variable.)  If ALIGNMENT is <=0,

returns to the default for this machine.  The struct alignment is in

addition to the alignment implied by the most restrictively aligned

member of the struct.  Most machines want ALIGNMENT of 1.

SEE ALSO: add_member

*/

structof

/* DOCUMENT structof(object)

returns the data type of OBJECT, or nil for non-array OBJECTs.

Use typeof(object) to get the ASCII name of a the data type.

SEE ALSO: typeof, dimsof, numberof, sizeof, nameof

*/

sum

/* DOCUMENT sum(x)

returns the scalar sum of all elements of its array argument.

SEE ALSO: avg, min, max

*/

sun3_primitives

/* DOCUMENT sun3_primitives, file

sets FILE primitive data types to be native to Sun-2 or Sun-3.

*/

sun_primitives

/* DOCUMENT sun_primitives, file

sets FILE primitive data types to be native to Sun, HP, IBM, etc.

*/

swrite

/* SEE write     */

symbol_def

/* DOCUMENT symbol_def(func_name)(arglist)

or symbol_def(var_name)

invokes the function FUNC_NAME with the specified ARGLIST,

returning the return value.  ARGLIST may be zero or more arguments.

In fact, symbol_def("fname")(arg1, arg2, arg3) is equivalent to

fname(arg1, arg2, arg3), so that "fname" can be the name of any

variable for which the latter syntax is meaningful -- interpreted

function, built-in function, or array.

Without an argument list, symbol_def("varname") is equivalent to

varname, which allows you to get the value of a variable whose name

you must compute.

DO NOT OVERUSE THIS FUNCTION.  It works around a specific deficiency

FROM symbol_def TO timer

of the Yorick language -- the lack of pointers to functions -- and

should be used for such purposes as hook lists (see openb).

SEE ALSO: symbol_set

*/

symbol_set

/* DOCUMENT symbol_set, var_name, value

is equivalent to the redefinition

varname= value

except that var_name="varname" is a string which must be computed.

DO NOT OVERUSE THIS FUNCTION.  It works around a specific deficiency

of the Yorick language -- the lack of pointers to functions, streams,

bookmarks, and other special non-array data types.

SEE ALSO: symbol_def

*/

system

/* DOCUMENT system, "shell command line"

Passes the command line string to a shell for execution.

If the string is constant, you may use the special syntax:

$shell command line

(A long command line may be continued by ending the line with \

as usual.)  The system function syntax allows Yorick to compute

parts of the command line string, while the simple $ escape

syntax does not.  In either case, the only way to get output

back from such a command is to redirect it to a file, then

read the file.  Note that Yorick does not regain control

until the subordinate shell finishes.  (Yorick will get control

back if the command line backgrounds the job.)

WARNING: If Yorick has grown to a large size, this may crash

your operating system, since the underlying POSIX fork function

first copies all of the running Yorick process before the exec

function can start the shell.  See Y_SITE/sysafe.i for a fix.

SEE ALSO: popen

*/

tan

/* SEE sin     */

tanh

/* SEE sinh     */

timer

/* DOCUMENT timer, elapsed

or timer, elapsed, split

updates the ELAPSED and optionally SPLIT timing arrays.  These

arrays must each be of type array(double,3); the layout is

[cpu, system, wall], with all three times measured in seconds.

ELAPSED is updated to the total times elapsed since this copy

of Yorick started.  SPLIT is incremented by the difference between

the new values of ELAPSED and the values of ELAPSED on entry.

This feature allows for primitive code profiling by keeping

FROM timer TO transpose

separate accounting of time usage in several categories, e.g.--

elapsed= total= cat1= cat2= cat3= array(double, 3);

timer, elapsed0;

elasped= elapsed0;

... category 1 code ...

timer, elapsed, cat1;

... category 2 code ...

timer, elapsed, cat2;

... category 3 code ...

timer, elapsed, cat3;

... more category 2 code ...

timer, elapsed, cat2;

timer, elapsed0, total;

The wall time is not absolutely reliable, owning to possible

rollover at midnight.

SEE ALSO: timestamp, timer_print

*/

timer_print

/* DOCUMENT timer_print, label1, split1, label2, split2, ...

or timer_print

or timer_print, label_total

prints out a timing summary for splits accumulated by timer.

timer_print, "category 1", cat1, "category 2", cat2,

"category 3", cat3, "total", total;

SEE ALSO: timer

*/

timestamp

/* DOCUMENT timestamp()

returns string of the form "Sun Jan  3 15:14:13 1988" -- always

has 24 characters.

SEE ALSO: timer

*/

transpose

/* DOCUMENT transpose(x)

or transpose(x, permutation1, permutation2, ...)

transpose the first and last dimensions of array X.  In the second

form, each PERMUTATION specifies a simple permutation of the

dimensions of X.  These permutations are compounded left to right

to determine the final permutation to be applied to the dimensions

of X.  Each PERMUTATION is either an integer or a 1D array of

integers.  A 1D array specifies a cyclic permutation of the

dimensions as follows: [3, 5, 2] moves the 3rd dimension to the

5th dimension, the 5th dimension to the 2nd dimension, and the 2nd

dimension to the 3rd dimension.  Non-positive numbers count from the

end of the dimension list of X, so that 0 is the final dimension,

-1 in the next to last, etc.  A scalar PERMUTATION is a shorthand

for a cyclic permutation of all of the dimensions of X.  The value

of the scalar is the dimension to which the 1st dimension will move.

Examples:  Let x have dimsof(x) equal [6, 1,2,3,4,5,6] in order

to be able to easily identify a dimension by its length. Then:

dimsof(x)                          == [6, 1,2,3,4,5,6]

FROM transpose TO updateb

dimsof(transpose(x))               == [6, 6,2,3,4,5,1]

dimsof(transpose(x,[1,2]))         == [6, 2,1,3,4,5,6]

dimsof(transpose(x,[1,0]))         == [6, 6,2,3,4,5,1]

dimsof(transpose(x,2))             == [6, 6,1,2,3,4,5]

dimsof(transpose(x,0))             == [6, 2,3,4,5,6,1]

dimsof(transpose(x,3))             == [6, 5,6,1,2,3,4]

dimsof(transpose(x,[4,6,3],[2,5])) == [6, 1,5,6,3,2,4]

*/

typeof

/* DOCUMENT typeof(object)

returns a string describing the type of object.  For the basic

data types, these are "char", "short", "int", "long", "float",

"double", "complex", "string", "pointer", "struct_instance",

"void", "range", "struct_definition", "function", "builtin",

"stream" (for a binary stream), and "text_stream".

SEE ALSO: structof, dimsof, sizeof, numberof, nameof

*/

uncen

/* DOCUMENT uncen(ptcen)

or uncen(ptcen, ireg)

returns zone centered version of the 2-D zone centered array PTCEN.

The result is (imax-1)-by-(jmax-1) if PTCEN is imax-by-jmax.

If the region number array IREG is specified, zones with region

number 0 are not included in the point centering operation.

Note that IREG should have dimensions imax-by-jmax, like

the input PTCEN array; the first row and column of IREG are ignored.

Without IREG, uncen(ptcen) is equivalent to ptcen(uncp,uncp).

Do not use uncen to zone center data which is naturally point

centered -- use the zncen function for that purpose.  The uncen

function is the (nearly) exact inverse of the ptcen function,

so that uncen(ptcen(zncen, ireg), ireg) will return the original

zncen array.  The uncen reconstruction is as exact as possible,

given the finite precision of floating point operations.

SEE ALSO: ptcen, zncen

*/

updateb

/* DOCUMENT file= updateb(filename)

or file= updateb(filename, primitives)

open a binary date file FILENAME for update (mode "r+b").

The optional PRIMITIVES argument is as for the createb function.

If the file exists, it is opened as if by openb(filename),

otherwise a new PDB file is created as if by createb(filename).

SEE ALSO: openb, createb, cd, save, restore, get_vars, get_addrs

close102, close102_default, open102, at_pdb_open, at_pdb_close

*/

FROM use_origins TO where

use_origins

/* DOCUMENT dummy= use_origins(dont_force)

Yorick array dimensions have an origin as well as a length.

By default, this origin is 1 (like FORTRAN arrays, unlike C

arrays).  However, the array function and the pseudo-index (-)

can be used to produce arrays with other origins.

Initially, the origin of an array index is ignored by Yorick; the

first element of any array has index 1.  You can change this

default behavior by calling use_origins with non-zero DONT_FORCE,

and restore the default behavior by calling use_origins(0).

When the returned object DUMMY is destroyed, either by return from

the function in which it is a local variable, or by explicit

redefintion of the last reference to it, the treatment of array

index origins reverts to the behavior prior to the call to

use_origins.  Thus, you can call use_origins at the top of a

function and not worry about restoring the external behavior

before every possible return (including errors).

SEE ALSO: array, dimsof, orgsof

*/

vax_primitives

/* DOCUMENT vax_primitives, file

sets FILE primitive data types to be native to VAXen, H-double, only.

*/

vaxg_primitives

/* DOCUMENT vaxg_primitives, file

sets FILE primitive data types to be native to VAXen, G-double, only.

*/

volume

/* DOCUMENT volume(r, z)

returns the zonal volumes of the 2-D cylindrical mesh (R, Z).

If R and Z are imax-by-jmax, the result is (imax-1)-by-(jmax-1).

The volume is positive when, say, Z increases with i and R increases

with j.  For example, volume([[0,0],[1,1]],[[0,1],[0,1]]) is +pi.

SEE ALSO: area

*/

warranty

/* SEE copyright     */

where

/* DOCUMENT where(x)

returns the vector of longs which is the index list of non-zero

values in the array x.  Thus, where([[0,1,3],[2,0,4]]) would

return [2,3,4,6].  If noneof(x), where(x) is a special range

function which will return a nil value if used to index an array;

hence, if noneof(x), then x(where(x)) is nil.

If x is a non-zero scalar, then where(x) returns a scalar value.

FROM where TO write

The rather recondite behavior for scalars and noneof(x) provides

maximum performance when the merge function to be used with the

where function.

SEE ALSO: where2, merge, merge2 allof, anyof, noneof, nallof, sort

*/

where2

/* DOCUMENT where2(x)

like where(x), but the returned list is decomposed into indices

according to the dimensions of x.  The returned list is always

2 dimensional, with the second dimension the same as the dimension

of where(x).  The first dimension has length corresponding to the

number of dimensions of x.  Thus, where2([[0,1,3],[2,0,4]]) would

return [[2,1],[3,1],[1,2],[3,2]].

If noneof(x), where2 returns [] (i.e.- nil).

SEE ALSO: where, merge, merge2, allof, anyof, noneof, nallof, sort

*/

write

/* DOCUMENT n= write(f, format=fstring, linesize=l, obj1, obj2, ...)

n= write(format=fstring, linesize=l, obj1, obj2, ...)

or strings= swrite(format=fstring, linesize=l, obj1, obj2, ...)

writes text to I/O stream F (1st form), or to the terminal (2nd

form), or to the STRINGS string array (3rd form), representing

arrays OBJ1, OBJ2, ..., according to the optional FSTRING.  The

optional linesize L defaults to 80 characters, and helps restrict

line lengths when FSTRING is not given, or does not contain

newline directives.  The write function always appends to the

end of a text file; the position for a sequence of reads is

not affected by intervening writes.

There must be one conversion specifier (see below) in FSTRING for

each OBJ to be written; the type of the conversion specifier must

generally match the type of the OBJ.  That is, an integer OBJ

requires an integer specifier (d, i, o, u, x, or c) in FSTRING,

a real OBJ requires a real specifier (e, f, or g), a string OBJ

requires the string specifier (s), and a pointer OBJ requires a

the pointer specifier (p).  An OBJ may not be complex, a structure

instance, or any non-array Yorick object.  If FSTRING is not

supplied, or if it has fewer conversion specifiers than the

number of OBJ arguments, then Yorick supplies default specifiers

(" %8ld" for integers, " %14.6lg" for reals, " %s" for strings, and

" %8p" for pointers).  If FSTRING contains more specifiers than

there are OBJ arguments, the part of FSTRING beginning with the

first specifier with no OBJ is ignored.

The OBJ may be scalar or arrays, but the dimensions of the OBJ

must be conformable.  If the OBJ are arrays, Yorick behaves as

if he write were called in a loop dimsof(OBJ1, OBJ2, ...) times,

writing one array element of each of the OBJ according to FSTRING

on each pass through the loop.  The swrite function returns a

string array with dimensions dimsof(OBJ1, OBJ2, ...).  The write

function inserts a newline between passes through the array if

the line produced by the previous pass did not end with a

newline, and if the total number of characters output since the

FROM write TO write

previous inserted newline, plus the number of characters about

to be written on the current pass, would exceed L characters

(L defaults to 80).  The write function returns the total

number of characters output.

The FSTRING is composed of a series of "directives" which are

(1) characters other than % -- copied directly to output

(2) conversion specifiers beginning with % and ending with a

character specifying the type of conversion -- specify

how to convert an OBJ into characters for output

The conversion specifier is of the form %FW.PSC, where:

F is zero or more optional flags:

- left justify in field width

+ signed conversion will begin with either + or -

(space) signed conversion  will begin with either space or -

# alternate form (see description of each type below)

0 pad field width with leading 0s instead of leading spaces

W is either a decimal integer specifying the minimum field width

(padded as specified by flags), or not present to use the

minimum number of characters required.

.P is either a decimal integer specifying the precision of the

result, or not present to get the default.  For integers, this

is the number of digits to be printed (possibly forcing leading

zeroes), and defaults to 1.  For reals, this is the number of

digits after the decimal point, and defaults to 6.  For strings,

this is the maximum number of characters to print, and defaults

to infinity.

S is either one of the characters 'h', 'l', or 'L', or not

present.  Yorick allows this for compatibility with the C

library functions, but ignores it.

C is a character specifying the type of conversion:

d, i  - decimal integer

o     - octal integer (# forces leading 0)

u     - unsigned decimal integer (same as d for Yorick)

x, X            - hex integer (# forces leading 0x)

f     - floating point real in fixed point notation

(# forces decimal)

e, E  - floating point real in scientific notation

g, G  - floating point real in fixed or scientific notation

depending on the value converted (# forces decimal)

s   - string of ASCII characters

c   - integer printed as corresponding ASCII character

p   - pointer

%   - the ordinary % character; complete conversion

specification must be "%%"

The write function is modeled on the ANSI standard C library

fprintf and sprintf functions, but differs in several respects:

(1) Yorick's write cannot handle the %n conversion specifier

in FSTRING.

(2) Yorick's write may insert additional newlines if the OBJ

are arrays, to avoid extremely long output lines.

SEE ALSO: print, exit, error, read, rdline, open, close, save, restore

*/

FROM xdr_primitives TO zncen

xdr_primitives

/* DOCUMENT xdr_primitives, file

sets FILE primitive data types to be XDR (external data representation).

*/

yorick_stats

/* DOCUMENT yorick_stats

returns an array of longs describing Yorick memory usage.

For debugging.  See ydata.c source code.

*/

zncen

/* DOCUMENT zncen(ptcen)

or zncen(ptcen, ireg)

returns zone centered version of the 2-D point centered array PTCEN.

The result is (imax-1)-by-(jmax-1) if PTCEN is imax-by-jmax.

If the region number array IREG is specified, zones with region

number 0 are not included in the point centering operation.

Note that IREG should have dimensions imax-by-jmax, like

the input PTCEN array; the first row and column of IREG are ignored.

Without IREG, zncen(ptcen) is equivalent to ptcen(zcen,zcen).

SEE ALSO: ptcen, uncen

*/